CO3 AT2

M. Srusty (192412611)

19. Decimation by Factor 2 Simulate decimation by a factor of 2 with anti-alias filtering and analyze the resulting signal spectrum. (25) (BL3) (WK4) Given Data: Sampling Frequency = 8 kHz  Signal Frequencies = 500 Hz and 2 kHz  Decimation Factor = 2  Tasks: Design anti-alias filter.  Filter the signal.  Perform decimation.  Plot spectra before and after decimation.  Verify alias prevention.  Discuss observations.

clc;

clear;

close all;

%% Given Parameters

Fs = 8000;              % Original sampling frequency = 8 kHz

F1 = 500;               % Signal frequency 1 = 500 Hz

F2 = 2000;              % Signal frequency 2 = 2 kHz

M = 2;                  % Decimation factor

%% Time Vector

duration = 0.02;        % Signal duration = 20 ms

t = 0:1/Fs:duration-1/Fs;

%% Generate Input Signal

x = sin(2*pi*F1*t) + sin(2*pi*F2*t);

%% Plot Input Signal

figure;

plot(t*1000, x, 'LineWidth', 1.2);

grid on;

xlabel('Time (ms)');

ylabel('Amplitude');

title('Original Signal');

%% ---------------------------------------------------------

% 1. Design Anti-Alias Low-Pass Filter

% ----------------------------------------------------------

% After decimation:

Fs_new = Fs/M;           % New sampling frequency = 4 kHz

Nyquist_new = Fs_new/2;  % New Nyquist frequency = 2 kHz

% Cutoff frequency selected below 2 kHz

Fc = 1500;               % Cutoff frequency = 1.5 kHz

% Design 6th-order Butterworth low-pass filter

N = 6;

[b,a] = butter(N, Fc/(Fs/2), 'low');

%% Frequency Response of Anti-Alias Filte

figure;

freqz(b,a,1024,Fs);

title('Anti-Alias Low-Pass Filter Response');

%% ---------------------------------------------------------

% 2. Filter the Input Signal

% ----------------------------------------------------------

x_filtered = filtfilt(b,a,x);

%% Plot Filtered Signal

figure;

plot(t*1000, x_filtered, 'LineWidth', 1.2);

grid on;

xlabel('Time (ms)');

ylabel('Amplitude');

title('Signal After Anti-Alias Filtering');

%% ---------------------------------------------------------

% 3. Perform Decimation

% ----------------------------------------------------------

% Downsample by factor of 2

x_decimated = x_filtered(1:M:end);

% New time vector

t_decimated = (0:length(x_decimated)-1)/Fs_new;

%% Plot Decimated Signal

figure;

plot(t_decimated*1000, x_decimated, 'LineWidth', 1.2);

grid on;

xlabel('Time (ms)');

ylabel('Amplitude');

title('Signal After Decimation by Factor 2');

%% ---------------------------------------------------------

% 4. Spectrum Before Filtering

% ----------------------------------------------------------

L = length(x);

X = fft(x);

f = (0:L-1)*(Fs/L);

% Single-sided spectrum

P2 = abs(X/L);

P1 = P2(1:floor(L/2)+1);

P1(2:end-1) = 2*P1(2:end-1);

f1 = f(1:floor(L/2)+1);

%% ---------------------------------------------------------

% 5. Spectrum After Anti-Alias Filtering

% ----------------------------------------------------------

X_filtered = fft(x_filtered);

P2_filtered = abs(X_filtered/L);

P1_filtered = P2_filtered(1:floor(L/2)+1);

P1_filtered(2:end-1) = 2*P1_filtered(2:end-1);

%% ---------------------------------------------------------

% 6. Spectrum After Decimation

% ----------------------------------------------------------

Ld = length(x_decimated);

X_decimated = fft(x_decimated);

fd = (0:Ld-1)*(Fs_new/Ld);

P2_decimated = abs(X_decimated/Ld);

P1_decimated = P2_decimated(1:floor(Ld/2)+1);

P1_decimated(2:end-1) = 2*P1_decimated(2:end-1);

fd1 = fd(1:floor(Ld/2)+1);

%% ---------------------------------------------------------

% 7. Plot Spectra

% ----------------------------------------------------------

figure;

subplot(3,1,1);

plot(f1/1000, P1, 'LineWidth', 1.2);

grid on;

xlabel('Frequency (kHz)');

ylabel('Magnitude');

title('Spectrum Before Filtering');

xlim([0 Fs/2/1000]);

subplot(3,1,2);

plot(f1/1000, P1_filtered, 'LineWidth', 1.2);

grid on;

xlabel('Frequency (kHz)');

ylabel('Magnitude');

title('Spectrum After Anti-Alias Filtering');

xlim([0 Fs/2/1000]);

subplot(3,1,3);

plot(fd1/1000, P1_decimated, 'LineWidth', 1.2);

grid on;

xlabel('Frequency (kHz)');

ylabel('Magnitude');

title('Spectrum After Decimation by 2');

xlim([0 Fs_new/2/1000]);

%% ---------------------------------------------------------

% 8. Display Important Values

fprintf('Original Sampling Frequency = %.2f kHz\n', Fs/1000);

fprintf('Decimation Factor = %d\n', M);

fprintf('New Sampling Frequency = %.2f kHz\n', Fs_new/1000);

fprintf('New Nyquist Frequency = %.2f kHz\n', Nyquist_new/1000);

fprintf('Anti-Alias Filter Cutoff = %.2f kHz\n', Fc/1000);

OUTPUT: