EXPERIMENT 6: Finite Word length Effects

EXPERIMENT 6.1: Uniform Quantization and Quantization Error Analysis of a Discrete-Time Sinusoidal Signal Using MATLAB

clc;

clear;

close all;

%% Signal

fs = 1000; f = 50;

t = 0:1/fs:0.1;

x = sin(2*pi*f*t);

%% Quantization Levels

L = [8 16 32];

colors = {'r','g','m'};

figure;

for i = 1:length(L)

% Quantization

xq{i} = round((x + 1) * (L(i) - 1) / 2) * (2 / (L(i) - 1)) - 1;

% Error & MSE

e{i} = x - xq{i};

mse(i) = mean(e{i}.^2);

% Plot Quantized Signals

subplot(4,1,i+1);

stairs(t,xq{i},colors{i},'LineWidth',1.2);

title(['Quantized Signal (' num2str(L(i)) ' Levels)']);

grid on;

end

% Original Signal

subplot(4,1,1);

plot(t,x,'LineWidth',1.5);

title('Original Signal'); grid on;

% Display MSE

fprintf('MSE for 8 Levels = %f\n', mse(1));

fprintf('MSE for 16 Levels = %f\n', mse(2));

fprintf('MSE for 32 Levels = %f\n', mse(3));

%% Error Plots

figure;

for i = 1:3

subplot(3,1,i);

plot(t,e{i});

title(['Error (' num2str(L(i)) ' Levels)']);

grid on;

end

EXPERIMENT: 6.2 Analysis of Finite Word Length Effects in Digital Signal Representation Using Truncation and Rounding Techniques in MATLAB

clc; clear; close all

n=0:79;

x=sin(2*pi*n/40);

q=8;

xt=floor(x*q)/q;

xr=round(x*q)/q;

et=x-xt;

er=x-xr;

% Fig 1

figure

plot(n,x,'b',n,xr,'r','LineWidth',1.5)

grid on

title('Original vs Quantized')

legend('Original','Quantized')

% Fig 2

figure

plot(n,x,'b',n,xt,'r',n,xr,'g','LineWidth',1.5)

grid on

title('Truncation vs Rounding')

legend('Original','Truncated','Rounded')

% Fig 3

figure

stem(n,et,'r')

hold on

stem(n,er,'g')

grid on

title('Quantization Error')

legend('Truncation','Rounding')

% Fig 4

figure

bar([mean(et.^2) mean(er.^2)])

set(gca,'XTickLabel',{'Truncation','Rounding'})

grid on

title('MSE Comparison')

ylabel('MSE')